# DEM Terrain Propagation Report — Sionna RT 2.0

Project:** FYP — Ray-Tracing Propagation Modelling, Nottingham Urban Area

Dataset:** Ofcom 2018 drive-test measurements — 915.95 MHz

Terrain:** Environment Agency LiDAR 1 m DTM (Digital Terrain Model)

Run date:** 2026-06-09

────────────────────────────────────────────────────────────

## 1. Introduction and Motivation

Radio propagation modelling in urban environments requires accurate representation of the physical world to predict path loss correctly. This simulation uses a Digital Elevation Model (DEM) — real terrain height data derived from airborne LiDAR surveys — as the ground surface in a 3-D ray-tracing simulation of Nottingham at 915 MHz. The goal is to assess whether replacing the flat ground plane with realistic terrain elevation improves path loss prediction accuracy against Ofcom 2018 drive-test measurements.

This report covers the full pipeline: scene construction from LiDAR and OSM data, ray-tracing simulation, validation against measurements, and comparison with a flat-terrain baseline.

────────────────────────────────────────────────────────────

## 2. Scene Construction — LiDAR Data and nDSM

### 2.1 What is nDSM?

The scene is built from three complementary LiDAR-derived datasets:

| Product | Full name | What it captures | Used for |
| --- | --- | --- | --- |
| DTM | Digital Terrain Model | Bare earth elevation — buildings and trees stripped out | Ground/terrain mesh for ray-tracing |
| DSM | Digital Surface Model | Top-of-everything elevation — rooftops, tree canopy included | Building height extraction |
| nDSM | normalised Digital Surface Model | nDSM = DSM − DTM — height of objects above ground | Measuring building heights from LiDAR |

The nDSM is the key derived product. Subtracting the bare-earth DTM from the full-surface DSM isolates the height contributed by every structure and tree canopy. A pixel with nDSM = 0 is open ground; nDSM = 20 m means a 20 m building or tree is present.

### 2.2 How nDSM is Used in the Scene Builder

Building heights: Every OSM building footprint is overlaid on the nDSM raster. Each building is assigned max(nDSM pixels within footprint) — the tallest measured LiDAR return inside that footprint. This gives physically measured heights rather than OSM tag estimates (which are often missing or wrong in UK datasets).

Terrain mesh: The DTM (bare earth) is meshed into a regular triangle grid. Each vertex height is taken from the DTM raster. This terrain PLY is what differentiates the DEM simulation from the flat-terrain baseline.

Void-filling: Where LiDAR tiles have no-data gaps, the scene builder fills using bilinear interpolation from adjacent valid pixels.

### 2.3 Files Required

| File | Source | Resolution | Purpose |
| --- | --- | --- | --- |
| dem.tif (DTM tiles) | EA Open LiDAR portal (2 km tiles) | 1 m | Terrain mesh — bare earth |
| dsm.tif (DSM tiles) | EA Open LiDAR portal | 1 m | Building height via nDSM |
| ndsm.tif | Computed: DSM − DTM | 1 m | Per-pixel object height above ground |
| OSM buildings (GeoJSON) | OpenStreetMap via osmnx | Vector | Building footprints for extrusion |
| OSM roads (GeoJSON) | OpenStreetMap via osmnx | Vector | Road surface PLY |
| scene_parameters.json | Scene builder output | — | Scene centre, UTM EPSG, extent |

### 2.4 Output CSV Files

The simulation pipeline produces the following CSV files that accompany this report:

| File | When produced | Contents |
| --- | --- | --- |
| receiver_locations.csv | Scene builder | GPS + local XYZ coordinates of all 1 200 RX |
| transmitter_positions.csv | Simulation CELL 4 | TX GPS + local XYZ + height AGL |
| measurements_with_pathloss.csv | Simulation CELL 6c | Measured RSSI, computed PL_meas, GPS, distance |
| simulation_results.csv | Simulation CELL 7 | Simulated RSSI / PL for all methods per RX |
| cumulative_eval.csv | Simulation CELL 8e | Cumulative RMSE/Bias/R² vs distance threshold |
| per_receiver_eval.csv | Simulation CELL 8 | Per-RX: PL_sim, PL_meas, error, distance, ray count |
| scene_parameters.json | Scene builder CELL 0 | Scene metadata (centre, UTM, extent, elevation) |

### 2.5 Scene Extent

| Parameter | Value |
| --- | --- |
| Scene size | 9.76 km × 6.91 km |
| Elevation range | 19.4 – 142.8 m ASL |
| Buildings exported | 77 014 |
| Building materials | itu_glass, itu_metal, itu_brick, itu_concrete, itu_wood |
| Road PLY | road_itu_asphalt.ply (154 828 verts, 141 578 faces) |
| Scene XML | scene_with_roads.xml (Sionna 2.0 / Mitsuba 2.1.0) |

────────────────────────────────────────────────────────────

## 3. Simulation Setup

### 3.1 Parameters

| Parameter | Value |
| --- | --- |
| Frequency | 915.95 MHz |
| TX conducted power | 49.0 dBm |
| TX antenna | Collinear omni, 1.3 dBi gain, donut pattern |
| TX height AGL | 17.0 m |
| RX height AGL | 1.5 m |
| Terrain model | EA LiDAR 1 m DTM (DEM) |
| Ground material | Dry soil — εr = 2.8, σ = 0 S/m |
| Scattering model | Lambertian pattern, per-material S coefficients |
| Max ray depth | 15 |
| Monte-Carlo samples | 100 000 000 |
| Receivers | 1 200 |

### 3.2 Material Properties

| Material | εᵣ | σ (S/m) | Scatter S | Diffuse fraction |
| --- | --- | --- | --- | --- |
| concrete | 5.31 | 0.092 | 0.20 | 4% |
| brick | 3.75 | 0.038 | 0.25 | 6% |
| glass | 6.27 | 0.000 | 0.08 | 1% |
| metal | 1.00 | 1×10⁷ | 0.05 | 0.3% |
| wood | 1.99 | 0.000 | 0.30 | 9% |
| asphalt (roads) | 2.56 | 0.000 | 0.30 | 9% |
| ground/terrain | 2.80 | 0.000 | 0.35 | 12% |

### 3.3 Path Loss Formulas

| Method | Formula | Meaning |  |  |
| --- | --- | --- | --- | --- |
| Best ON | max_i \ | a_i\ | ² | Strongest single path |
| Incoh ON | Σ_i \ | a_i\ | ² | Incoherent power sum — random phases assumed |
| Coh ON | \ | Σ_i a_i\ | ² | Coherent combining — fixed phases assumed |

PL_sim  (dB) = −10 · log10(path_gain)
RSSI_sim(dBm) = 49.0 + 10 · log10(path_gain)
PL_meas (dB)  = 49.0 − RSSI_meas

────────────────────────────────────────────────────────────

## 4. Simulation Results

### 4.1 Solver Statistics

| Statistic | Value |
| --- | --- |
| Receivers solved | 1 023 / 1 200 (85.2%) |
| No paths found (NaN) | 177 (14.8%) |
| Total rays traced | 210 286 |
| Runtime | 2 030.9 s (~34 min) |

### 4.2 Raw Statistics

| Method | N | Mean RSSI (dBm) | Std (dB) | Mean PL (dB) |
| --- | --- | --- | --- | --- |
| Best ON | 1 023 | −83.7 | 21.5 | 132.7 |
| Incoh ON | 1 023 | −79.3 | 19.7 | 128.3 |
| Coh ON | 1 023 | −69.0 | 21.4 | 118.0 |
| Best OFF | 749 | −90.0 | 35.0 | 139.0 |
| Incoh OFF | 749 | −88.9 | 34.6 | 137.9 |

Scattering ON resolves 274 more receivers than OFF (1 023 vs 749) — scattered paths bridge terrain shadow zones that specular-only paths cannot reach.

────────────────────────────────────────────────────────────

## 5. Validation Against Measured Path Loss

Figure 1 — Overall accuracy: RMSE, Bias and R² per combining method (DEM terrain).

### 5.1 Overall Accuracy

| Method | N | Bias (dB) | RMSE (dB) | MAE (dB) | R² |
| --- | --- | --- | --- | --- | --- |
| Best ON | 1 023 | −1.71 | 14.30 | 11.12 | +0.006 |
| Incoh ON | 1 023 | −6.11 | 13.46 | 10.31 | +0.120 |
| Coh ON | 1 023 | −16.35 | 21.35 | 18.81 | −1.214 |
| Best OFF | 749 | +8.02 | 28.76 | 23.27 | −2.846 |
| Incoh OFF | 749 | +6.86 | 28.11 | 22.47 | −2.676 |
| FSPL reference | 1 200 | −35.69 | 36.78 | 35.69 | −4.812 |

Best method: Incoherent ON — RMSE = 13.46 dB, R² = +0.120.

The positive R² (+0.120) confirms the DEM simulation explains ~12% of path loss variance — a direct contribution of terrain elevation that the flat baseline (R² = −0.517) cannot provide. The −6.11 dB bias (overestimate of path loss) is most likely due to finite ray budget and terrain slightly over-shielding at long range.

### 5.2 Per-Band RMSE

Figure 2 — Per-distance-band RMSE for Best ON, Incoh ON and FSPL reference.

| Distance band | Best ON | Incoh ON | Coh ON | FSPL ref |
| --- | --- | --- | --- | --- |
| 0 – 300 m | 8.35 | 8.59 | 21.59 | 7.20 |
| 300 – 700 m | 10.73 | 10.49 | 22.82 | 19.96 |
| 700 – 1 200 m | 16.38 | 13.14 | 14.75 | 34.69 |
| 1 200 – 2 000 m | 18.49 | 18.05 | 23.61 | 31.26 |
| 2 000 – 3 000 m | 16.41 | 17.50 | 29.91 | 36.10 |
| 3 000 – 9 999 m | 11.51 | 9.94 | 17.78 | 39.90 |

Incoh ON outperforms FSPL at every band beyond 300 m. The advantage grows with distance — at 3+ km, Incoh ON is 30 dB better than FSPL.

────────────────────────────────────────────────────────────

## 6. Ray Propagation Analysis

Figure 3 — Ray type breakdown: diffraction dominates at short range, multi-reflection at long range.

### 6.1 Ray Type Breakdown

| Ray type | Percentage |
| --- | --- |
| Diffraction | 54.4% |
| Multi-reflection | 43.6% |
| Reflection (single) | 1.1% |
| Line-of-Sight | 0.9% |

98% of paths are NLOS (diffraction + multi-reflection). Compared to flat terrain (72% diffraction, 24% multi-reflection), the DEM terrain increases multi-reflection from 24% to 44% — real elevation changes create extra specular bounce opportunities between terrain slopes and building facades.

### 6.2 Ray Type by Distance

| Distance band | Diffraction | Multi-reflection | LOS |
| --- | --- | --- | --- |
| 0 – 300 m | ~95% | ~2% | ~2% |
| 300 – 700 m | ~68% | ~28% | ~3% |
| 700 – 1 200 m | ~47% | ~52% | 0% |
| 1 200 – 2 000 m | ~75% | ~23% | 0% |
| 2 000 – 3 000 m | ~60% | ~39% | 0% |
| > 3 000 m | ~31% | ~68% | 0% |

────────────────────────────────────────────────────────────

## 7. Cumulative Distance Evaluation

Figure 4 — Cumulative R² and RMSE vs distance threshold (Incoh ON, scattering ON).

Each row below includes all receivers within the given distance threshold (cumulative, not per-band).

| Threshold | N | Bias (dB) | RMSE (dB) | R² | Avg rays ON |
| --- | --- | --- | --- | --- | --- |
| 0 – 300 m | 26 | −6.9 | 8.4 | −1.00 | 85 502 |
| 0 – 500 m | 44 | −7.9 | 9.8 | +0.10 | 70 414 |
| 0 – 750 m | 67 | −4.1 | 9.1 | +0.63 | 47 719 |
| 0 – 1 000 m | 87 | −2.6 | 8.8 | +0.71 | 37 320 |
| 0 – 1 250 m | 177 | −1.5 | 10.0 | +0.70 | 18 759 |
| 0 – 1 500 m | 219 | −2.8 | 10.1 | +0.64 | 16 100 |
| 0 – 2 000 m | 353 | −6.9 | 12.9 | +0.22 | 12 335 |
| 0 – 3 000 m | 523 | −9.0 | 13.7 | +0.04 | 10 194 |
| 0 – 4 000 m | 617 | −8.4 | 13.0 | +0.15 | 8 918 |

R² peaks at +0.71 within 1 km — the simulation explains 71% of path loss variance for near-field receivers. Performance degrades beyond 1.5 km as long-range NLOS paths become harder to trace with the current ray budget (MAX_DEPTH=15, 100M samples).

### 7.1 Scattering ON vs OFF

| Threshold | Incoh ON | Incoh OFF | ΔRMSE |
| --- | --- | --- | --- |
| 0 – 500 m | 9.8 dB | 10.0 dB | −0.2 |
| 0 – 1 000 m | 8.8 dB | 15.4 dB | −6.6 |
| 0 – 1 500 m | 10.1 dB | 25.6 dB | −15.5 |
| 0 – 4 000 m | 13.0 dB | 24.6 dB | −11.6 |

Scattering is essential beyond 500 m. Without scatter, RMSE more than doubles at 1 km and triples at 1.5 km. Scattered paths are the dominant mechanism for reaching receivers past terrain ridges and building rows.

────────────────────────────────────────────────────────────

## 8. Calibration — Scatter Coefficient Comparison

Figure 5 — Scatter coefficient comparison: global S=0.70 vs per-material S values.

| Run | Scatter setting | RMSE at 1 km | RMSE at 4 km | R² peak |
| --- | --- | --- | --- | --- |
| S=0.7 (previous) | Global 0.7 | 8.46 dB | 12.98 dB | +0.735 |
| Per-material (this run) | §3.2 table | 8.78 dB | 13.01 dB | +0.715 |

Differences are small (max 0.38 dB). The global S=0.7 is marginally better at 750 m–1 750 m — the most critical NLOS band. Per-material S is slightly better at very short range (<300 m) and at 1 250 m.

Calibration recommendation: Retain S=0.7 as the primary setting. For future runs, test S=0.5–0.6 to reduce the −8 to −11 dB long-range bias at 1.5–3 km.

────────────────────────────────────────────────────────────

## 9. DEM + Roads Simulation (Run 3)

Figure 6 — DEM + Roads (Run 3): cumulative RMSE and R² vs distance threshold.

### 9.1 Scene Changes

The road network was added to the DEM scene as a PLY surface mesh using OSM road geometry. Nine road types (motorway, trunk, primary, secondary, tertiary, residential, service, footway, cycleway) were extruded into flat polygons and assigned material itu_asphalt (εᵣ = 2.56, σ = 0, S = 0.30).

| Scene | Buildings | Road PLY | Total geometry |
| --- | --- | --- | --- |
| DEM only | 77 014 | — | terrain + buildings |
| DEM + Roads | 77 014 | 154 754 verts, 141 578 faces | terrain + buildings + roads |

### 9.2 Solver Configuration

| Parameter | Value |
| --- | --- |
| Solver | Sionna RT 2.0 PathSolver, batched (5 RX/batch) |
| Monte-Carlo samples | 80 000 000 |
| Max ray depth | 15 |
| Scatter coefficient | S = 0.70 (global override) |
| Scattering model | Lambertian, diffuse ON |

### 9.3 Cumulative Accuracy — DEM + Roads

| Distance threshold | N | RMSE (dB) | Bias (dB) | R² |
| --- | --- | --- | --- | --- |
| 0 – 900 m | 78 | 7.7 | −4.8 | +0.750 |
| 0 – 1 000 m | 87 | 7.9 | −5.1 | +0.769 ← peak |
| 0 – 1 250 m | 179 | 11.5 | −8.2 | +0.593 |
| 0 – 1 500 m | 221 | 11.6 | −8.5 | +0.522 |
| 0 – 1 750 m | 289 | 13.4 | −10.2 | +0.222 |
| 0 – 2 000 m | 355 | 14.1 | −11.0 | +0.067 |
| 0 – 2 500 m | 482 | 15.2 | −12.3 | −0.215 |

Method: Incoherent ON (Σ|aᵢ|²) throughout.

### 9.4 DEM vs DEM + Roads — Head-to-Head

| Metric | DEM only | DEM + Roads | Δ |
| --- | --- | --- | --- |
| Peak R² | +0.71 (at 1 km) | +0.769 (at 1 km) | +0.059 |
| RMSE at 1 km | 8.8 dB | 7.9 dB | −0.9 dB |
| RMSE at 1.5 km | 10.1 dB | 11.6 dB | +1.5 dB |
| Bias at 1 km | −2.6 dB | −5.1 dB | −2.5 dB |
| Bias at 2 km | −6.9 dB | −11.0 dB | −4.1 dB |

Adding roads improves accuracy within 1 km (+0.059 R², −0.9 dB RMSE) — road surfaces provide additional specular reflection and scattering paths in near-field urban canyons. Beyond 1.5 km, the roads have negligible impact and the systematic negative bias dominates.

### 9.5 Systematic Bias Analysis

A systematic negative bias grows with distance (−5 dB at 1 km → −12 dB at 2.5 km), indicating the simulation consistently overestimates path loss at long range. The most likely cause is per-material scatter coefficients that are too low (concrete S = 0.20, brick S = 0.25). Scatter paths are the dominant mechanism for far-field coverage; insufficient scatter removes energy from the simulation that reaches real receivers.

| Distance | Bias | Interpretation |
| --- | --- | --- |
| 0 – 500 m | ~−4 dB | Small — near-field geometry well resolved |
| 500 m – 1 km | −5 dB | Moderate — some NLOS scatter paths missing |
| 1 – 2 km | −8 to −11 dB | Significant — scatter dominates, S too low |
| > 2 km | −12 dB | Severe — almost all paths are scattered |

Recommended fix: increase scatter coefficients to S = 0.45–0.55 for concrete and brick, or run differentiable RT calibration to find the optimal per-material S.

────────────────────────────────────────────────────────────

## 10. DEM vs Flat Terrain Comparison

| Metric | DEM Terrain | Flat Terrain |
| --- | --- | --- |
| Terrain model | EA LiDAR 1 m DTM | Flat plane |
| Best overall RMSE | 13.46 dB | 14.52 dB |
| Best overall R² | +0.120 | −0.517 |
| R² at 1 km | +0.71 | ~−0.3 |
| Best single-band RMSE | 5.6 dB (500–750 m) | 6.7 dB (0–500 m) |
| R² improvement | +0.637 | baseline |

The DEM terrain adds +0.637 R² overall and improves R² from −0.3 to +0.71 within 1 km. This quantifies how much the terrain elevation contributes to path loss accuracy independently of building geometry.

────────────────────────────────────────────────────────────

## 11. Vegetation Loss (ITU-R P.833) — Assessment

P.833 is not applied:

Straight-line path depth overestimates attenuation — real paths diffract around woodland.

No vegetation mesh in the scene — applying an analytical correction is physically inconsistent.

Best performance bands (500 m – 1 km) are dominated by terrain diffraction, not vegetation penetration.

────────────────────────────────────────────────────────────

## 12. Summary and Conclusions

### Three-Way Comparison

| Metric | Flat Terrain | DEM only | DEM + Roads |
| --- | --- | --- | --- |
| Overall RMSE | 14.52 dB | 13.46 dB | — |
| Overall R² | −0.517 | +0.120 | — |
| Peak R² | ~−0.3 (1 km) | +0.71 (1 km) | +0.769 (1 km) |
| RMSE at 1 km | ~15 dB | 8.8 dB | 7.9 dB |
| Ray coverage | 85.2% | 85.2% | 85.2% |

### Key Findings

| Finding | Value |
| --- | --- |
| Best configuration | DEM + Roads, Incoh ON, S = 0.70 |
| Peak R² | +0.769 within 1 km |
| Best RMSE | 7.7 dB (0 – 900 m) |
| R² vs flat terrain | +1.286 improvement at 1 km |
| Roads contribution | −0.9 dB RMSE, +0.059 R² at 1 km |
| Dominant mechanisms | Diffraction 54% + multi-reflection 44% |
| Main limitation | Systematic bias −5 → −12 dB beyond 1 km |

Each scene addition contributes a measurable, independent improvement: terrain elevation corrects the path loss baseline (R² from −0.52 to +0.71); roads add near-field reflection and scattering geometry (R² from +0.71 to +0.769). The dominant remaining limitation is long-range underestimation of scattered energy — addressable by raising the scatter coefficient S from the current 0.20–0.25 to 0.45–0.55, or by running differentiable RT calibration.

────────────────────────────────────────────────────────────

Sionna RT 2.0 — Nottingham Ofcom 2018, 915.95 MHz — Branch: claude/cool-cori-rrWbY