# Flat Terrain Simulation Report — Sionna RT 2.0

Scene: Nottingham / Ofcom 2018 — flat terrain (z = 0 plane)

Frequency: 915.95 MHz

TX conducted power: 49.0 dBm

TX antenna gain: 1.3 dBi (collinear omni, donut pattern)

RX height AGL: 1.5 m

MAX_DEPTH: 8

NUM_SAMPLES_PS: 2 000 000

SCATTER_OVERRIDE: 0.9 (all materials, amplitude split; diffuse power fraction = S² = 81%)

Receivers: 1 200

PL formula: PL_sim = −10·log10(path_gain)

RSSI formula: RSSI_sim = TX_CONDUCTED_DBM + 10·log10(path_gain) = 49.0 + 10·log10(path_gain)

PL reference (measured): PL_meas = 49.0 − RSSI_meas

## CELL 7 — Path Solver Results

Config: MAX_DEPTH=15 | 50M samples | batch=5 | all mechanisms ON

Receivers solved: 992 / 1200 (82.7%) | Zero-path (NaN): 208 | Total rays: 153 450

PL formula: PL_sim = −10·log10(path_gain) | PL_meas = 49.0 − RSSI_meas

### Overall Metrics — All Receivers

| Method | N | Bias (dB) | MSE (dB²) | RMSE (dB) | MAE (dB) | STD (dB) | R² |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Best ON | 992 | −4.35 | 385.14 | 19.63 | 14.77 | 19.14 | −0.517 |
| Incoh ON | 992 | −8.30 | 428.07 | 20.69 | 15.85 | 18.95 | −0.686 |
| Coh ON | 992 | −15.02 | 691.57 | 26.30 | 21.16 | 21.59 | −1.724 |
| Best OFF | 695 | +1.08 | 1059.44 | 32.55 | 26.64 | 32.53 | −3.378 |
| Incoh OFF | 695 | −0.92 | 1097.75 | 33.13 | 27.20 | 33.12 | −3.536 |
| Coh OFF | 695 | +3.14 | 923.82 | 30.39 | 23.93 | 30.23 | −2.817 |
| FSPL ref | 1200 | −35.69 | 1353.23 | 36.79 | 35.69 | 8.92 | −4.812 |

### Per-Band RMSE (dB)

| Band | Best ON | Incoh ON | Coh ON | Best OFF | Incoh OFF | Coh OFF | FSPL ref |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0–200 m | 8.03 | 8.12 | 13.70 | 8.03 | 8.12 | 8.09 | 7.17 |
| 200–500 m | 11.77 | 13.12 | 25.40 | 10.33 | 12.89 | 10.33 | 12.62 |
| 500–1000 m | 26.59 | 28.51 | 41.33 | 26.63 | 28.53 | 21.73 | 26.82 |
| 1000–1500 m | 32.92 | 36.45 | 46.62 | 32.95 | 36.34 | 26.67 | 34.03 |
| 1500–2000 m | 19.76 | 22.12 | 26.00 | 24.80 | 26.63 | 20.47 | 31.87 |
| 2000–3000 m | 14.16 | 13.23 | 15.56 | 33.86 | 33.69 | 33.55 | 36.12 |
| 3000–9999 m | 15.63 | 15.31 | 18.86 | 37.94 | 36.88 | 37.46 | 39.90 |

### Ray Classification

Total rays: 153 450 from 992 resolved receivers

| Ray type | Count | % of total |
| --- | --- | --- |
| DIFFRACTION | 110 618 | 72.1% |
| MULTI_REFLECTION | 37 006 | 24.1% |
| REFLECTION | 3 190 | 2.1% |
| LOS | 2 636 | 1.7% |

**72% diffraction dominant** — flat terrain with dense urban buildings makes diffraction the primary propagation mechanism at all ranges. MULTI_REFLECTION (24%) increases beyond 2 km as rays bounce between building faces to reach deep NLOS. LOS is only 1.7% confirming the predominantly NLOS nature of the drive route.

### Key Findings

Best ON is the overall best method (RMSE 19.6 dB, Bias −4.4 dB) — lower than incoh ON because flat terrain's ray explosion (avg 13k–104k rays/RX) inflates the incoherent power sum with weak diffuse paths.

Scatter OFF collapses beyond 1 km — only 695 receivers resolved vs 992 with scatter ON. RMSE 30–33 dB for all OFF methods at all ranges. Scattered paths are essential for coverage beyond 1 km even on flat terrain.

Coh OFF best at 500–1500 m (RMSE 21.7 and 26.7 dB) — the coherent sum suppresses the noise from the ray explosion; without scatter, coherent OFF benefits from clean specular-only paths.

FSPL is a competitive reference at 0–200 m (RMSE 7.17 dB vs Best ON 8.03 dB) — near-TX, the urban overhead above free space is small and consistent.

All methods degrade at 500–1500 m — this is where the flat terrain geometry fails most severely. Real Nottingham terrain provides 8–15 dB additional shielding at these ranges that the flat scene cannot reproduce.

Good recovery at 2–4 km — Best ON and Incoh ON recover to RMSE ~13–16 dB because at these ranges, multiple diffraction paths converge and the model better captures the statistical average even without terrain.

## CELL 8e — Cumulative Distance Evaluation (Scattering ON vs OFF)

Three combining methods evaluated per distance threshold:

| Symbol | Method | Description |
| --- | --- | --- |
| incoh | Incoherent | `path_gain = Σ\ |
| coh | Coherent | `path_gain = \ |
| best | Best path | `path_gain = max(\ |

**Note on avg_rays:** Scattering ON produces 60 000–76 000 rays/RX vs ~450–560 rays/RX with scattering OFF.

The ray explosion on flat terrain (no terrain shielding, open ground scatter) makes the incoherent ON sum physically unreliable — energy is accumulated over tens of thousands of paths. Coherent OFF is therefore the recommended primary metric for this scene.

### Results

| Band | N | avg_rays ON | avg_rays OFF | Method | Bias (dB) | RMSE (dB) | R² |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0–100 m | 8 | 76 180 | 564 | ON incoh | −8.9 | 10.3 | −7.014 |
|  |  |  |  | OFF incoh | −8.9 | 10.3 | −7.022 |
|  |  |  |  | ON coh | −7.9 | 10.0 | −6.475 |
|  |  |  |  | OFF coh | −7.3 | 9.7 | −6.013 |
|  |  |  |  | ON best | −8.6 | 10.2 | −6.781 |
|  |  |  |  | OFF best | −8.6 | 10.2 | −6.781 |
| 0–200 m | 17 | 71 458 | 462 | ON incoh | −5.5 | 7.5 | −3.365 |
|  |  |  |  | OFF incoh | −5.5 | 7.5 | −3.368 |
|  |  |  |  | ON coh | −2.8 | 7.1 | −2.921 |
|  |  |  |  | OFF coh | −1.9 | 7.2 | −3.002 |
|  |  |  |  | ON best | −4.6 | 7.2 | −2.979 |
|  |  |  |  | OFF best | −4.6 | 7.2 | −2.979 |
| 0–300 m | 26 | 72 488 | 399 | ON incoh | −6.6 | 8.1 | −0.869 |
|  |  |  |  | OFF incoh | −6.7 | 8.2 | −0.919 |
|  |  |  |  | ON coh | −3.7 | 7.1 | −0.447 |
|  |  |  |  | OFF coh | −1.5 | 6.5 | −0.207 |
|  |  |  |  | ON best | −5.1 | 7.0 | −0.387 |
|  |  |  |  | OFF best | −5.1 | 7.0 | −0.387 |
| 0–500 m | 44 | 71 510 | 431 | ON incoh | −9.5 | 11.2 | −0.163 |
|  |  |  |  | OFF incoh | −9.5 | 11.2 | −0.167 |
|  |  |  |  | ON coh | −6.3 | 10.0 | 0.060 |
|  |  |  |  | OFF coh | −2.3 | 6.7 | 0.580 |
|  |  |  |  | ON best | −7.3 | 9.2 | 0.209 |
|  |  |  |  | OFF best | −7.3 | 9.2 | 0.209 |
| 0–750 m | 67 | 62 890 | 451 | ON incoh | −13.4 | 16.2 | −0.198 |
|  |  |  |  | OFF incoh | −13.4 | 16.2 | −0.202 |
|  |  |  |  | ON coh | −11.4 | 16.4 | −0.232 |
|  |  |  |  | OFF coh | −6.5 | 11.1 | 0.434 |
|  |  |  |  | ON best | −11.4 | 14.6 | 0.031 |
|  |  |  |  | OFF best | −11.4 | 14.6 | 0.031 |
| 0–900 m | 78 | 61 322 | 458 | ON incoh | −15.7 | 18.6 | −0.453 |
|  |  |  |  | OFF incoh | −15.7 | 18.7 | −0.458 |
|  |  |  |  | ON coh | −13.7 | 18.3 | −0.408 |
|  |  |  |  | OFF coh | −7.3 | 11.4 | 0.457 |
|  |  |  |  | ON best | −13.5 | 16.7 | −0.166 |
|  |  |  |  | OFF best | −13.5 | 16.7 | −0.166 |
| 0–1000 m | 87 | 62 134 | 461 | ON incoh | −17.7 | 21.0 | −0.627 |
|  |  |  |  | OFF incoh | −17.7 | 21.0 | −0.630 |
|  |  |  |  | ON coh | −15.8 | 20.6 | −0.567 |
|  |  |  |  | OFF coh | −8.6 | 12.5 | 0.419 |
|  |  |  |  | ON best | −15.4 | 18.8 | −0.312 |
|  |  |  |  | OFF best | −15.4 | 18.8 | −0.312 |

### Best method summary — Coherent OFF

| Band | N | Bias (dB) | RMSE (dB) | R² |
| --- | --- | --- | --- | --- |
| 0–100 m | 8 | −7.3 | 9.7 | −6.013 |
| 0–200 m | 17 | −1.9 | 7.2 | −3.002 |
| 0–300 m | 26 | −1.5 | 6.5 | −0.207 |
| 0–500 m | 44 | −2.3 | 6.7 | 0.580 |
| 0–750 m | 67 | −6.5 | 11.1 | 0.434 |
| 0–900 m | 78 | −7.3 | 11.4 | 0.457 |
| 0–1000 m | 87 | −8.6 | 12.5 | 0.419 |

### Key findings

Best operating range: 0–500 m — coherent OFF delivers RMSE = 6.7 dB, R² = 0.580, bias = −2.3 dB (slight underestimate).

Coherent OFF consistently outperforms all other methods at every distance threshold. The coherent sum suppresses the artificial ray-count amplification introduced by scatter ON.

Scattering ON degrades accuracy for flat terrain. With S = 0.9 (81% diffuse power fraction) and no terrain shielding, the open ground generates ~150× more rays than scatter OFF (avg 62k–76k vs 450–560). Incoherent power summing over 60k+ paths is physically unrealistic; it inflates the received power estimate by >15 dB at ranges >500 m.

Best-path ON/OFF are identical — scatter does not change the dominant ray, only adds weak diffuse paths.

Performance degrades beyond 500 m due to the flat terrain geometry (no Nottingham hills to shadow distant paths), causing systematic underestimation of path loss at mid-range.

## CELL P.833 — Vegetation Excess Loss

OSM tags: landuse=forest/wood, natural=wood (dense woodland only — parks/gardens/meadow excluded)

Downloaded: 273 features → 271 UTM polygons

Frequency: 0.9160 GHz

### Raw output

| Band | N | Mean veg loss (dB) | Max veg loss (dB) | N_veg% |
| --- | --- | --- | --- | --- |
| 0–300 m | 26 | 0.00 | 0.00 | 0.0% |
| 300–500 m | 18 | 0.00 | 0.00 | 0.0% |
| 500–750 m | 23 | 0.00 | 0.00 | 0.0% |
| 750–1000 m | 20 | 3.32 | 7.33 | 50.0% |
| 1000–1250 m | 92 | 12.79 | 25.27 | 97.8% |
| 1250–1500 m | 42 | 21.66 | 30.77 | 100.0% |
| 1500–2000 m | 134 | 27.10 | 58.92 | 100.0% |
| 2000–3000 m | 170 | 27.67 | 55.45 | 100.0% |
| 3000–9999 m | 675 | 22.17 | 67.33 | 100.0% |

Receivers with woodland depth > 0: 1121 / 1200

Mean veg depth (all RX): 140.6 m

Max veg depth: 826.1 m

Mean Weissberger loss: 21.21 dB

Max Weissberger loss: 67.33 dB

### Assessment

The tags are correct and the download is fresh (woodland only). The results reflect real woodland in Nottingham's outskirts — Sherwood Forest edge, country parks and woodland belts that OSM correctly maps as forest/wood. The 675 receivers at 3–10 km all have 100% woodland hit, pulling the scene-wide mean up to 140.6 m depth.

However the correction is not applicable for this scenario for two reasons:

Straight-line assumption breaks at long range. Real rays diffract over and around woodland rather than cutting straight through 140 m of canopy. The straight-line depth severely overestimates actual attenuation at >1 km.

The evaluation range is unaffected. At 0–750 m (the range where coh OFF achieves RMSE = 6.7 dB and R² = 0.58) woodland coverage is 0% — P.833 has zero effect on these results.

| Range | P.833 effect | Verdict |
| --- | --- | --- |
| 0–750 m | 0 dB | No change — correction not triggered |
| 750–1000 m | 3.32 dB mean | Marginal, possibly valid |
| >1000 m | 12–27 dB | Overcorrects — straight-line assumption invalid |

Conclusion: P.833 is not applied for the flat terrain report. The CELL 8e results (coh OFF, 0–500 m) stand without vegetation correction.

## Configuration

| Parameter | Value |
| --- | --- |
| FLAT_TERRAIN | True |
| GROUND_PRESET | dry (er=2.8, σ=0) |
| SCATTER_OVERRIDE | 0.9 |
| MAX_DEPTH | 8 |
| NUM_SAMPLES_PS | 2 000 000 |
| Receivers | 1 200 |
| TX_CONDUCTED_DBM | 49.0 dBm |