# GPS → Mitsuba Coordinate Transformation

Sionna 0.19.2  ·  Nottingham Ray-Tracing Simulation

# Overview

Every GPS coordinate from the Ofcom drive-test CSV must be converted into a 3-D position (x, y, z) in Mitsuba's local scene space before it can be used as a receiver or transmitter. The transformation has three stages:

| Stage | Input | Output | Tool |
| --- | --- | --- | --- |
| 1 — GPS → UTM | lon/lat (degrees) | easting/northing (m, absolute) | pyproj EPSG:4326→32630 |
| 2 — UTM → Local | easting/northing (m) | x/y (m, scene-centred) | subtract bbox centre |
| 3 — Z from mesh | local x/y (m) | z (m, DSM surface height) | Mitsuba ray_cast |

# Stage 1 — WGS84 (GPS) → UTM Zone 30N

Input: longitude/latitude in decimal degrees (EPSG:4326, WGS84) — the raw format stored in nottingham3602.csv.

Output: easting (X) and northing (Y) in absolute metres, UTM Zone 30N (EPSG:32630). At UK latitude, 1° longitude ≈ 67 km and 1° latitude ≈ 111 km.

from pyproj import Transformer

gps_to_utm = Transformer.from_crs("EPSG:4326", "EPSG:32630", always_xy=True)

# Example — TX site

lon, lat = -1.2559, 52.9863

easting, northing = gps_to_utm.transform(lon, lat)

# → easting  = 457,842 m

# → northing = 5,873,415 m

ℹ  always_xy=True forces (longitude, latitude) order regardless of CRS axis convention — without this pyproj may silently swap the axes.

# Stage 2 — UTM Absolute → Scene-Local (centred at 0, 0)

Mitsuba does not know about geographic coordinates. The scene PLY meshes are built relative to the centre of the scene bounding box, so all positions must use the same origin.

## Origin computation (done once at scene build time)

# Scene bbox midpoint in WGS84

center_lon = (SCENE_WEST + SCENE_EAST)  / 2   # e.g. -1.327

center_lat = (SCENE_SOUTH + SCENE_NORTH) / 2  # e.g. 52.960

# Convert to UTM to get the metric origin

utm_center_x, utm_center_y = gps_to_utm.transform(center_lon, center_lat)

# utm_center_x ≈ 453,475 m

# utm_center_y ≈ 5,870,463 m

## Per-point conversion

def gps_to_local(lon, lat):

ux, uy = gps_to_utm.transform(lon, lat)

local_x = ux - utm_center_x   # metres east  of scene centre

local_y = uy - utm_center_y   # metres north of scene centre

return local_x, local_y

# Example — drive-test point

local_x, local_y = gps_to_local(-1.2508, 52.9875)

# → local_x = +4,711 m  (east  of scene centre)

# → local_y = +3,074 m  (north of scene centre)

ℹ  The TX at (-1.2559, 52.9863) maps to approximately (4711 m, 3074 m) in local coordinates — it is near the north-east edge of the scene, not at the origin.

# Stage 3 — Z Height from Mitsuba Scene Mesh (ray_cast)

The terrain mesh was built from the AWS DSM (Digital Surface Model — includes building rooftops and canopy). To place a receiver or transmitter consistently with this mesh, a downward ray is fired from z = 2000 m and the intersection height is returned.

import mitsuba as mi

def ray_cast_ground_z(x, y, max_height=2000.0):

"""Return scene-local Z of ground/surface at (x, y)."""

ray = mi.Ray3f(

mi.Point3f(float(x), float(y), max_height),  # origin

mi.Vector3f(0.0, 0.0, -1.0)                  # direction: straight down

)

si = scene.mi_scene.ray_intersect(ray)

if bool(si.is_valid()):

return float(si.p.z)   # Z of first surface hit

return 0.0                  # fallback if no intersection

# Example

ground_z = ray_cast_ground_z(4711.0, 3074.0)

# → ground_z = 67.1 m  (DSM surface = terrain + any building)

# Final receiver position

rx_z = ground_z + 1.5    # 1.5 m antenna height AGL  → 68.6 m

# Final transmitter position

tx_z = ground_z + 17.0   # 17 m mast height AGL       → 84.1 m

ℹ  Why ray_cast and not DEM tiles? The DTM (bare-earth) tiles give ~65 m at the TX location while the DSM terrain mesh reads ~131 m ASL (67 m local). Using DTM would place the TX 49 m underground. ray_cast is always consistent with the loaded scene geometry.

# Complete Example — One Drive-Test Point

| Step | Operation | Value |
| --- | --- | --- |
| CSV input | Raw Ofcom measurement | lon=-1.2508, lat=52.9875, RSSI=-46.2 dBm |
| Stage 1 | GPS → UTM 30N | easting=458,186 m northing=5,873,537 m |
| Stage 2 | UTM → local (subtract centre) | local_x=+4,711 m local_y=+3,074 m |
| Stage 3 | ray_cast Z at (4711, 3074) | ground_z = 67.1 m |
| RX height | ground_z + 1.5 m AGL | rx_z = 68.6 m |
| Mitsuba pos. | Receiver position vector | [4711.0, 3074.0, 68.6] metres |

# Reference Frames Summary

| Frame | EPSG | Origin | Units | Used for |
| --- | --- | --- | --- | --- |
| WGS84 | 4326 | Earth centre | degrees | CSV input, GPS |
| UTM 30N | 32630 | Zone SW corner | metres abs | Intermediate (pyproj) |
| Scene-local | — | Scene bbox centre | metres rel | Mitsuba, PLY, all outputs |
| Mitsuba Z | — | Scene origin plane | metres | DSM surface from ray_cast |

# DSM vs DTM — Why It Matters

The AWS elevation-tiles-prod bucket provides DSM (Digital Surface Model): elevation includes buildings, trees, and other above-ground objects. The scene terrain mesh is built from DSM tiles.

| Model | Measures | At TX site (Nottingham) | Effect if used for Z |
| --- | --- | --- | --- |
| DSM | Surface (buildings + terrain) | ~131 m ASL / 67 m local | TX at correct height ✓ |
| DTM | Bare earth only | ~65 m ASL / 1 m local | TX 49 m underground ✗ |

ℹ  Solution: always use ray_cast_ground_z() — it fires a ray into the loaded Mitsuba scene (built from DSM) so the returned Z is guaranteed to match the scene geometry, regardless of which elevation dataset was used.